# Network school project

🏫 1️⃣ First: Understand the Building Structure

- Many departments

- Many PCs

- Many servers

- One building

In real life, we design using a hierarchical model:

Internet

|

Router

|

Core Switch

|

-------------------------

|    |    |    |    |   |

Access Access Access Access Server

SW1    SW2    SW3    SW4   SW

This is called:

### 🏗 Three-Tier Architecture (Simplified)

- Core Layer

- Access Layer

- End Devices

2️⃣ What Devices To Place (Exact List)

### 🔹 1 Router (2911)

This connects to the Internet later.

### 🔹 1 Core Switch (3560 Layer 3)

This is the brain of the network.

### 🔹 5 Access Switches (2960)

Each one will serve departments.

### 🔹 1 Server Switch (2960)

3️⃣ How To Organize On Screen

Place devices like this:

## TOP CENTER

Router

## Below Router (Center)

Core Switch

## Left Side (Access Switches)

Access SW1
Access SW2
Access SW3
Access SW4
Access SW5

Right Side

Server Switch

# Cable Types (Very Important)

Use:

### PC → Switch

Copper Straight-Through

### Access Switch → Core Switch

Copper Straight-Through (Gigabit port)

### Core Switch → Router

Copper Straight-Through

# Tip: How To Check If Cabling Is Correct

After connecting:

- All links should turn green

- If orange → wait 30 seconds (STP)

- If red → wrong port or wrong cable

# Data link layer(Layer 2)

Now we move from Physical Layer (Layer 1) to Data Link Layer (Layer 2).

Now the network starts becoming intelligent.

We are going to configure VLANs on the CORE SWITCH (3560).

VLANs must exist on:

- ✅ Core Switch

- ✅ All Access Switches

- ✅ Server Switch

But it starts with the Core.

Go to:

CORE-SW → CLI

## 🔹 1️⃣ Enter Configuration Mode

## 🔹 2️⃣ Create VLANs One by One

# 🔎 3️⃣ Verify VLAN Creation

# 🚨 Important Concept

Right now:

✔ VLANs exist
 ❌ But no ports belong to them yet
 ❌ No routing yet
 ❌ No communication yet

We just created logical containers.

Think of it like:

We built rooms, but no one lives inside yet.

# IMPORTANT CONCEPT FIRST

There are 2 types of switch ports:

### 🔹 Access Port

- Belongs to ONE VLAN

- Used for PCs, printers, servers

### 🔹 Trunk Port

- Carries MULTIPLE VLANs

- Used between switches

Right now we configure ACCESS PORTS.

# Assign VLANs to Access Switch Ports

We DO NOT assign PC ports on the Core.

We configure them on:

- ACC-SW1

- ACC-SW2

- ACC-SW3

- ACC-SW4

- ACC-SW5

- SERVER-SW

Access Switch 1 (Management VLAN 10)

Owners, Directors, Lawyers

Now suppose:

- Fa0/1–Fa0/2 → Owners (2 PCs)

- Fa0/3–Fa0/9 → Directors (5 PCs)

Both belong to VLAN 10 (MANAGEMENT)

Instead of doing one by one, use interface range command:

Type on the CLI:

interface range fa0/1 - 9

switchport mode access

switchport access vlan 10

Access Switch 2 (ADMIN VLAN 20)

Example:

- HR + Accounting + Marketing PCs on Fa0/1 – Fa0/8

Type on the CLI:

interface range fa0/1 - 8

switchport mode access

switchport access vlan 20

Access Switch 3 (ACADEMIC VLAN 30)

Teachers + Library

Type on the CLI:

interface range fa0/1 - 8

switchport mode access

switchport access vlan 30

Access Switch 4

Restaurant → VLAN 60

interface range fa0/1 - 2

switchport mode access

switchport access vlan 60

Access Switch 5

Students → VLAN 40

interface range fa0/1 - 4

switchport mode access

switchport access vlan 40

Access Switch 6

IT → VLAN 50

interface range fa0/1 - 4

switchport mode access

switchport access vlan 50

Access Switch 7

Law_department → VLAN 70

interface range fa0/1 - 3

switchport mode access

switchport access vlan 70

Server Switch

All servers belong to VLAN 200

interface range fa0/1 - 10

switchport mode access

switchport access vlan 200

# Configure TRUNK Ports

Right now:

✔ VLANs created
 ✔ Access ports assigned

But switches cannot pass VLAN traffic between each other yet.

We must configure trunk ports on:

- Each Access Switch uplink

- The Core Switch ports that connect to Access Switches

- The Server Switch uplink

# 🧠 What Is a Trunk?

A trunk port:

- Carries multiple VLANs

- Connects switches together

- Does NOT belong to one VLAN

# STEP 8 — Configure Trunks (Access Switch Side)

On each Access Switch:

Find which port connects to the Core (example: Gi0/1).

Then configure:

interface gig0/1

switchport mode trunk

That’s it.

Repeat on:

- ACC-SW1

- ACC-SW2

- ACC-SW3

- ACC-SW4

- ACC-SW5

- ACC-SW6

- ACC-SW7

- ACC-SW8

- SERVER-SW

# STEP 9 — Configure Trunks on Core Switch

Now go to CORE-SW:

For each port connecting to access switches:

Example:

interface range gigabitEthernet1/0/1 - 7

switchport mode trunk

(Adjust the range to match your topology.)

# Verify

After configuring, run the IOS command:

show interfaces trunk

You should see something like:

Port        Mode     Encapsulation  Status    Native vlan

Gi1/0/1     on       802.1q         trunking  1

If you see "trunking" → perfect.

# STEP 10 — Create VLAN Interfaces (SVIs)

# Where Should VLAN Interfaces (SVIs) Be Created?

You must create them on the Core Switch (3560 Multilayer Switch).

In your topology the device is:

CORE_SWITCH_EN24

Why?

Because the Core switch is performing the routing between VLANs.

So it acts like a router for all VLAN networks.

# 🏗 What a VLAN Interface (SVI) Does

Each VLAN needs a gateway so devices can send traffic outside their network.

Example:

Students PC:

# Configuration on CORE_SWITCH_EN24

Execute the following command to each interface vlan below:

### VLAN 10

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

### VLAN 20

interface vlan 20

ip address 192.168.20.1 255.255.255.0

no shutdown

### VLAN 30

interface vlan 30

ip address 192.168.30.1 255.255.255.0

no shutdown

### VLAN 40

interface vlan 40

ip address 192.168.40.1 255.255.255.0

no shutdown

### VLAN 50

interface vlan 50

ip address 192.168.50.1 255.255.255.0

no shutdown

### VLAN 60

interface vlan 60

ip address 192.168.60.1 255.255.255.0

no shutdown

### VLAN 70

interface vlan 70

ip address 192.168.70.1 255.255.255.0

no shutdown

### VLAN 200

interface vlan 200

ip address 192.168.200.1 255.255.255.0

no shutdown

# 🔎 Verify Configuration

Run this command:

show ip interface brief

You should see something like:

Vlan10   192.168.10.1   up   up

Vlan20   192.168.20.1   up   up

Vlan30   192.168.30.1   up   up

...

Vlan200  192.168.200.1  up   up

If you see up/up, everything is correct.😎

# 🚨 Important Note

If an SVI shows:

up/down

or

down/down

It usually means:

- No active ports exist in that VLAN yet.

But this will fix itself once PCs are active.

# Do the DHCP configuration on PCs

Next we will do the DHCP configuration so PCs receive IP addresses automatically.

# Recommended for Your Project

Activate DHCP on the Core Switch (3560).

Why?

Because:

- Your Core switch is already doing inter-VLAN routing

- It is the default gateway for all VLANs

- This is common in small/medium enterprise networks

So DHCP and routing stay in the same device.

# 🧠 Network Logic

Your traffic flow now looks like this:

PC → Access Switch → Core Switch (Gateway + DHCP) → Other VLANs / Servers

The Core switch will:

- Assign IP addresses

- Provide gateway

- Route between VLANs

# STEP 3 — Configure DHCP on the CORE SWITCH

Go to:

CORE_SWITCH_EN24

Enter CLI:

enable

configure terminal

# First — Exclude Gateway Addresses

We must prevent DHCP from assigning the gateway IP.

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

ip dhcp excluded-address 192.168.40.1

ip dhcp excluded-address 192.168.50.1

ip dhcp excluded-address 192.168.60.1

ip dhcp excluded-address 192.168.70.1

ip dhcp excluded-address 192.168.200.1

# Now Create DHCP Pools

### VLAN 10 – Management

ip dhcp pool MANAGEMENT

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

### VLAN 20 – Admin

ip dhcp pool ADMIN

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

### VLAN 30 – Academic

ip dhcp pool ACADEMIC

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 8.8.8.8

### VLAN 40 – Students

ip dhcp pool STUDENTS

network 192.168.40.0 255.255.255.0

default-router 192.168.40.1

dns-server 8.8.8.8

### VLAN 50 – IT

ip dhcp pool IT

network 192.168.50.0 255.255.255.0

default-router 192.168.50.1

dns-server 8.8.8.8

### VLAN 60 – Services

ip dhcp pool SERVICES

network 192.168.60.0 255.255.255.0

default-router 192.168.60.1

dns-server 8.8.8.8

### VLAN 70 – Law

ip dhcp pool LAW

network 192.168.70.0 255.255.255.0

default-router 192.168.70.1

dns-server 8.8.8.8

⚠️ Do NOT create DHCP for VLAN 200 (Servers)

Servers should have static IP addresses.

# Configure DHCP

Now go to a PC:

Desktop → IP Configuration → DHCP(manually)

It should receive something like:

IP address: 192.168.X.X

Gateway: 192.168.X.1

DNS: 8.8.8.8

# Verify DHCP on Core

Run:

show ip dhcp binding

This shows which PCs received IP addresses.

# After DHCP Works

Next step will be:

Testing Inter-VLAN communication

We will test:

PC (HR) → PC (Teachers)

PC (IT) → Server

Students → Library

This proves the network is fully working.

When you finish the DHCP configuration and run the follow command to check the interfaces:

show ip interface brief

Then we’ll bring the network fully alive.